| Dip SW.1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1coin 1play | Off | Off | Off |  |  |  |  |  |
| 1coin 2play | On | Off | Off |  |  |  |  |  |
| 1coin 3play | Off | On | Off |  |  |  |  |  |
| 1coin 4play | On | On | Off |  |  |  |  |  |
| 1coin 5play | Off | Off | On |  |  |  |  |  |
| 2coin 1play | On | Off | On |  |  |  |  |  |
| 3coin 1play | Off | On | On |  |  |  |  |  |
| 4coin 1play | On | On | On |  |  |  |  |  |
| Normal |  |  |  |  |  |  | Off |  |
| Continue |  |  |  |  |  |  | On |  |
| Dip SW. 2 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Difficult 1 | Off | Off | Off |  |  |  |  |  |
| Difficult 2 | On | Off | Off |  |  |  |  |  |
| Difficult 3 | Off | On | Off |  |  |  |  |  |
| Difficult 4 | On | On | Off |  |  |  |  |  |
| Difficult 5 | Off | Off | On |  |  |  |  |  |
| Difficult 6 | On | Off | On |  |  |  |  |  |
| Difficult 7 | Off | On | On |  |  |  |  |  |
| Difficult 8 | On | On | On |  |  |  |  |  |
| Easy |  |  |  | Off | Off |  |  |  |
| Normal |  |  |  | On | Off |  |  |  |
| Hard |  |  |  | Off | On |  |  |  |
| Hardest |  |  |  | On | On |  |  |  |
| 2 player |  |  |  |  |  |  | Off | Off |
| 3 player |  |  |  |  |  |  | On | Off |
| 4 player |  |  |  |  |  |  | Off | On |
| Dip SW.3 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 2 lives | Off | Off |  |  |  |  |  |  |
| 3 lives | On | Off |  |  |  |  |  |  |
| 4 lives | Off | On |  |  |  |  |  |  |
| 1 life | On | On |  |  |  |  |  |  |
| Normal |  |  | Off |  |  |  |  |  |
| Free play |  |  | On |  |  |  |  |  |
| Normal play |  |  |  | Off |  |  |  |  |
| Pause |  |  |  | On |  |  |  |  |
| Flip no |  |  |  |  | Off |  |  |  |
| Flip yes |  |  |  |  | On |  |  |  |
| Demo sound off |  |  |  |  |  | Off |  |  |
| Demo sound on |  |  |  |  |  | On |  |  |
| Continue off |  |  |  |  |  |  | Off |  |
| Continue on |  |  |  |  |  |  | On |  |
| Game mode |  |  |  |  |  |  |  | Off |
| Test mode |  |  |  |  |  |  |  | On |