| DIP SW.1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1coin 1play | Off | Off | Off |  |  |  |  |  |
| 1coin 2play | On | Off | Off |  |  |  |  |  |
| 1coin 3play | Off | On | Off |  |  |  |  |  |
| 1coin 4play | On | On | Off |  |  |  |  |  |
| 1coin 6play | Off | Off | On |  |  |  |  |  |
| 2coin 1play | On | Off | On |  |  |  |  |  |
| 3coin 1play | Off | On | On |  |  |  |  |  |
| 4coin 1play | On | On | On |  |  |  |  |  |
| Always |  |  |  | Off | Off | Off |  |  |
| Normal |  |  |  |  |  |  | Off |  |
| Continue coin |  |  |  |  |  |  | On |  |
| Always |  |  |  |  |  |  |  | Off |
| DIP SW.B | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Easy 1 | Off | Off | Off |  |  |  |  |  |
| Easy 2 | On | Off | Off |  |  |  |  |  |
| Easy 3 | Off | On | Off |  |  |  |  |  |
| Normal 4 | On | On | Off |  |  |  |  |  |
| Normal 5 | Off | Off | On |  |  |  |  |  |
| Normal 6 | On | Off | On |  |  |  |  |  |
| Hard 7 | Off | On | On |  |  |  |  |  |
| Hard 8 | On | On | On |  |  |  |  |  |
| Easy 1 |  |  |  | Off | Off | Off |  |  |
| Easy 2 |  |  |  | On | Off | Off |  |  |
| Easy 3 |  |  |  | Off | On | Off |  |  |
| Normal 4 |  |  |  | On | On | Off |  |  |
| Normal 5 |  |  |  | Off | Off | On |  |  |
| Normal 6 |  |  |  | On | Off | On |  |  |
| Hard 7 |  |  |  | Off | On | On |  |  |
| Hard 8 |  |  |  | On | On | On |  |  |
| 3 chute |  |  |  |  |  |  | Off |  |
| 1 chute |  |  |  |  |  |  | On |  |
| 3player mode |  |  |  |  |  |  |  | Off |
| 2player mode |  |  |  |  |  |  |  | On |
| DIP SW.C | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 2players | Off | Off |  |  |  |  |  |  |
| 3players | On | Off |  |  |  |  |  |  |
| 4players | Off | On |  |  |  |  |  |  |
| 1players | On | On |  |  |  |  |  |  |
| Normal |  |  | Off |  |  |  |  |  |
| Free play |  |  | On |  |  |  |  |  |
| Screen pause on |  |  |  | Off |  |  |  |  |
| Screen pause off |  |  |  | On |  |  |  |  |
| Normal |  |  |  |  | Off |  |  |  |
| Flip screen |  |  |  |  | On |  |  |  |
| Demo sound off |  |  |  |  |  | Off |  |  |
| Demo sound on |  |  |  |  |  | On |  |  |
| Continue off |  |  |  |  |  |  | Off |  |
| Continue on |  |  |  |  |  |  | On |  |
| Game mode |  |  |  |  |  |  |  | Off |
| Test mode |  |  |  |  |  |  |  | On |