| DIP SW.1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 player | Off | Off |  |  |  |  |  |  |
| 2 player | On | Off |  |  |  |  |  |  |
| 4 player | Off | On |  |  |  |  |  |  |
| 5 player | On | On |  |  |  |  |  |  |
| Normal |  |  | Off |  |  |  |  |  |
| Hard |  |  | On |  |  |  |  |  |
| Energy normal |  |  |  | Off |  |  |  |  |
| Energy less |  |  |  | On |  |  |  |  |
| Coin 1 |  |  |  |  |  |  |  |  |
| 1coin 1play |  |  |  |  | Off | Off |  |  |
| 2coin 1play |  |  |  |  | On | Off |  |  |
| 3coin 1play |  |  |  |  | Off | On |  |  |
| 5coin 1play |  |  |  |  | On | On |  |  |
| Coin 2 |  |  |  |  |  |  |  |  |
| 1coin 2play |  |  |  |  |  |  | Off | Off |
| 1coin 3play |  |  |  |  |  |  | On | Off |
| 1coin 5play |  |  |  |  |  |  | Off | On |
| 1coin 6play |  |  |  |  |  |  | On | On |
| DIP SW.2 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Normal screen | Off |  |  |  |  |  |  |  |
| Reverse screen | On |  |  |  |  |  |  |  |
| Table |  | Off |  |  |  |  |  |  |
| Upright |  | On |  |  |  |  |  |  |
| Always |  |  | On |  |  |  |  |  |
| Demo sound on |  |  |  | Off |  |  |  |  |
| Demo sound off |  |  |  | On |  |  |  |  |
| Continue on |  |  |  |  | Off |  |  |  |
| Continue off |  |  |  |  | On |  |  |  |
| Stop mode off |  |  |  |  |  | Off |  |  |
| Stop mode on |  |  |  |  |  | On |  |  |
| Death mode off |  |  |  |  |  |  | Off |  |
| Death mode on |  |  |  |  |  |  | On |  |
| Always |  |  |  |  |  |  |  | Off |